# Logger Pro Notes for use with motion sensor

## To start collecting data:

- Click the Collect button on the toolbar at the top of the screen.

- Data collection will automatically stop after a set time, or you can stop it manually by clicking that button again.

## To adjust the data collection time:

- From the top menu bar go to Experiment—>Data Collection.

- Adjust Duration.

## To adjust the number of points used to calculate the velocity:

- File -> Settings for Untitled (or Settings for [filename])

- Change:

  - Number of Points for Derivative Calculation

  - Number of Points for Smoothing Calculation:

- The default values are 7. A value of 2 would be taking the slope of each pair of data points. The minimum you can set is 3. Smaller numbers will reduce the effect of the software’s smoothing to show more details about the individual measurements.

## To adjust sampling rate (how much data the sensor takes in per unit time):

- From the top menu bar go to Experiment—>Data Collection.

- Adjust Sampling Rate.

## To put multiple graphs on one set of axes:

- Right click on the graph.

- Click Graph Options.

- Select the data you want under Y-Axis Columns.

## To create a fit line:

- Select some area of the graph by clicking and dragging.

- On the toolbar at the top of the screen click on Linear fit (or Curve Fit).

## To show uncertainties on fit results:

- Double-click on fit results box.

- Select Show Uncertainty.